Assessment of Irrigation Potential & Investigation of Impact on the Abaya-Chamo Lakes

Seleshi B. Awulachew

IWMI

P.O.Box 5689, Addis Ababa, Ethiopia

Abstract:

In this paper, the existing water resources use for various purposes have been investigated, and the impact of use of water such as irrigation on the lake water volume have been evaluated based on various scenarios of development targets. The paper shortly highlights methodologies for potential estimation and water demands under limited data situation. Rainfall-runoff model, regionalization, water balance model and analyzed data are used to generate data and investigate impact of water use. Through impact study using lake water balance model, it is shown that not the consumptive use of irrigation that heavily affect the natural lakes of Abaya and Chamo, but the impact of erosion of the catchment area and sedimentation in the lakes has serious detrimental impact on the two lakes.

Key words: Irrigation potential, water balance, environmental impact, LAKEBAL

# Introduction

In the Southern part of Rift Valley Region of Ethiopia there are two natural Lakes, namely Abaya and Chamo Lakes. The watershed of the Lakes is about 18,500km² including water area and currently inhabited by more than 3 million people with growth rate of about 3% per annum. These Lakes are important shallow Lakes and among others importance, to inhabit a number of aquatic animals, as a source of fish and to locally modify the climate of the area. Although, not much is known about the two lakes, yet there are evidences that they are heavily influenced by human activities. They are endangered Lakes unless proper precautionary measures are undertaken through studying the impact of various activities. In order to investigate the available water resources quantity in the lakes as well as impacts and influences of natural and man-induced factors a water balance model of the two Lakes have been developed.

The knowledge of water balance of lakes and reservoirs is an essential component of water management. Water management decision as far as possible should be based on a thorough quantitative understanding of the hydrologic cycle of the lakes/reservoirs in the basin. In developing country like Ethiopia, the quantitative understanding of individual components of the hydrological system themselves is difficult.

This paper focuses on the use of developed water balance model for the two lakes in order to see the impact of irrigation development under various scenarios of extent of development. While, the importance of irrigation for poverty alleviation is clear, as discussed in the following section in the rift valley region, the impact of irrigation development on the environment, especially on the lake water volume and level is given emphasis and investigated.

The developed information is used for the simulation of lake water level. Furthermore, impacts of various influences such as sediment and water use are described and expected life of the Lakes under sediment loads are investigated and measures to reverse the deteriorating situation are briefly discussed.

# The Need and Use of Irrigation in the Study Area

## Water Use and Irrigation in Ethiopia

The existing water resource uses in Ethiopia in general and the study area in particular can be viewed in the following main aspects:

- Water supply for domestic, livestock and industrial uses

- Water use for irrigation

- Water use for Hydropower

Although the above uses have been also studied for developing water balance model of the lakes, the water use for irrigation development is reviewed here under.

Irrigation development could improve the agricultural productivity and enhance socioeconomic development of the bulk of Ethiopian community, through growth of production. According to English (1997) irrigation generally increases yields by 100% to 400%. Though, the majority of Ethiopian population's livelihood is dependent on agriculture, yet agriculture is practiced in a traditional and very inefficient way. Farming practice is dependent on rain fed agriculture and couldn't cope up with increasing population and dwindling land resource. The rain fed dependent agriculture, due to failure of rain, has time and again betrayed the farming community of Ethiopian people and exposed them to innumerable consequences.

The purpose of irrigation development is thus to improve the output in agricultural sector upon which 84% of the population of Ethiopia is dependent, which accounts 45-50 percent of GDP, and whose annual average growth rate is only at 0.75%, which is very low, see also Dessalegn (1999). Ethiopia's water resources use for irrigation is very limited. Ethiopia's irrigation potential estimate is not yet accurately known and one gets variable data. There have been various estimates. The potential estimates vary between 1 to 1.5 Mha according to old estimate by the World Bank (1973) to about 3.7 Mha by WAPCOS (1995). See also other estimates by MoA (1986), IFAD (1987), etc. It can be said that the country has got more than 3.5Mha (about 3.5% of land) that can be utilized for irrigation scheme. However, by 1994 only about 160,000 hectare, which is about 3% of developable potential, was under use, ESTC (1994). Today, the emphasis to small-scale and micro irrigation is high and almost all regions' endeavor is to improve agriculture through irrigation means. Thus, the above mentioned, existing irrigation of 3% of irrigable potential could be exceeded.

Just in year 2002/2003, millions of people (about 1 in 4 persons) needed emergency food aid in Ethiopia, because of mainly dependence on rain fed agriculture and lack of irrigation development. This is a consequence largely due to failure of crops in the sequential harvest seasons before and during the year 2003, which in turn associated to erratic rainfall.

## Existing Irrigation Development Activities in the Abaya-Chamo Sub-Basin (ACB)

In the ACB, the rain-fed sector development, like other parts of Ethiopia, is intensively cultivated but with low productivity. Abebe (2000) has investigated a certain area of the  study region (the Arbaminch area) using dry-spell analysis, for two types of crops and found out that almost every year dry-spell duration that causes damage to crop yield occurs in the area. The demand for irrigation development and thereby the need to increase productivity and secure food self sufficiency is a highly paramount issue and one of the most top priorities of the study area like other parts of Ethiopia.

Halcrow and Partners (1992) assessed the rain-fed agricultural potential of the Rift Valley Lakes Basin (RVLB). The use of irrigated farming in the Abaya-Chamo basin is practiced both as state owned and farmers owned, and irrigation is fairly accustomed in the low land areas. The following Table 1 provides summary of existing irrigation schemes related to ACB area. Figure_1 provide the GIS integrated mapped view of most of the existing irrigation schemes in the Abaya-Chamo Basin. In the ACB, the existing scheme thus could be considered to be about 8,000 ha. The existing water demand can't be accurately provided; as such data are almost not recorded.

| No | River | Sizable Irrigation Scheme | Irrigated Area (ha) |
| --- | --- | --- | --- |
| 1 | Bilate | Tobacco Farm Chericho | 1,000 1,300 |
| 2 | Gidabo | Gidabo diversion Wamole | 400 50 |
| 3 | Hare | Hare Farmers Irrigation | 1,200 |
| 4 | Baso | Baso Project | (small, estimates is not available) |
| 5 | Wajifo | Wajifo project Shafe project | 400 |
| 6 | Amesa | Humbo- Amesa irrigation | (small, estimates is not available) |
| 7 | Alluvial Fans of Abaya | Abaya State Farm | 700 |
| 8 | Kulfo | Arbaminch State Farm | 1,200 reduced to 800 |
| 9 | Sile | Sile State Farm | 1,300 |
| 10 | Argoba (Wezeka) | Argoba irrigation | 80 |
|  | Total |  | 7,630 +(unestimated) |

Table 1: Existing Major Irrigation Schemes in the Abaya-Chamo Basin, not including rain water harvesting and micro irrigation

On the other hand population of the region in the area considered here is increasing very high, and without intensive and high productivity of agriculture utilizing the low land areas, the already highly stressed land resource in some areas such as in Sodo Zuria, Damot Gale, Damot Weyde, Kembata woredas can't cope with such growth of population.

Irrigation has been practiced in the study region to a limited scale by farmers as small scale schemes and by state on medium and large-scale scheme basis. The latter has little to do with the welfare of the farmers, who are directly victimized because of climatic/rainfall variations. Irrigation owned by the farming community should be enhanced to contain the food shortage and related crisis. Irrigation development in study basin has also other tangible value beyond securing existence through food production. Through irrigation development, farmers could improve their means of income by producing cash crops. Such practice has highly changed the level of income of the communities who have already used irrigation. Particularly, some of the inhabitants of low land areas of Southwestern settlers/inhabitants in the Abaya drainage basin around Kollashara to Wajifo area are users of isolated and small scale irrigation practice.

Beyond the contribution to food security, the practice of irrigation in particular reference to Abaya Lake has positive contribution to the environment of the Lake. The use of irrigation and expansion of irrigation practice reduces the expansion of agricultural land which otherwise would have based on rain fed agriculture that would allow only one cropping season, and unreliable production.

# The Irrigation Potential and Water Demand Assessment Methodology & Result

  - Theoretical Potentials Areas for Irrigation

In the previous section existing irrigation of the study area was reviewed and mapped. In this section, potential sites, which in theory could be developable, are identified. Figure 1 provides map of these potential sites. In order to identify the potential sites, the GIS and map information are used to locate areas with small slopes (slope about <0.02) and lying in the vicinity of major rivers. The small slopes are taken as criteria, keeping in mind that the possible irrigation scheme is by gravity furrow/flood irrigation type. The nearness to the rivers is useful to reduce the conveyance system (irrigation canals length) and thereby to develop the irrigation system economically. Other necessary criteria such as soil suitability, water quality, etc., are not included for theoretical potential estimation. This assessment doesn't also include other possibilities such as small parcel of lands along the small tributaries draining to large rivers, which are very useful for micro and small irrigation. Developing such areas is obviously very important to enhance isolated and individual peasant based irrigation. On the other hand the identified areas could be owned not only by entrepreneurs or government but also most importantly by farmers as well. In the latter case parcel of land from the medium/large scheme could be allotted to individuals and water control and management could be handled in an organized manner.

Table 2 provides the theoretically developable potential irrigation size in the various rivers of the study area. The total area identified is summed to be 86,112 hectares of land which represent about 4.7% of the total area. This represents the theoretical/gross irrigation potential of the ACB along the major courses of the rivers, without considering detailed irrigation suitability. In the above potential estimate, 73,999 hectares is to be found in Abaya-Lakes watershed and the remaining 12,113 in Chamo Lake watershed.

  - Pre-identified Potential Areas

These are areas, which have been identified by various previous project studies, and their preliminary studies already exist. This includes areas of Gelana Irrigation Project (see Gibbs, A. & Partners(1987)), Gidabo Irrigation Project (see Tahl and Metaferia (1998)), Extension of Bilate Farm studied for MoWR whose studies are carried out in certain details. Table 3 provides the summary of size of land of major identified schemes under these various projects.

| River | Theoretical Irrigable Area (ha) | Remark |
| --- | --- | --- |
| Bilate: Upper Middle Lower | 6,637 |  |
|  | 2,008 |  |
|  | 28,649 |  |
| Gidabo | 10,308 | Middle & lower Gidabo |
| Gelana: Upper Middle and Lower | 1,058 |  |
|  | 18,271 |  |
| Kulfo | 1,809 | Substantial area could be overtaken by infrastructure development and expansion of Arbaminch town |
| Hare | 2,650 |  |
| Basso | 2,520 |  |
| Irae | 1,550 |  |
| Amessa | 348 |  |
| Sile and Sego | 5,295 |  |
| Wezeka, Arguba-Wezeka and Okotte Wezeka | 4,004 | The lower valley at vicinity of Chamo can be developed using the three rivers. |
| Doyasso | 1,005 |  |
| Total | 86,112 |  |

Table 2: Theoretical Irrigation Potential of ACB.

Figure 1: Existing and Potential Irrigation Sites in ACB

| S.No | Studied Scheme | Size of Land (ha) | Crops Identified |
| --- | --- | --- | --- |
| 1 | Bilate | 8,000 | Cotton, Maize, Tobacco, Pulses, Vegetables |
| 2 | Gelana | 11,000 | Cotton, Wheat, Maize, Pulses |
| 3 | Gidabo | 6,000 | Cotton, Corn, Peanuts, Melon, Mango, Coffee |
|  | Total | 25,000 |  |

Table 3: Previously identified, investigated but not yet fully implemented irrigation schemes

  - Estimation of Water Demand for Irrigation Use

In order to assess the water demand for the exploitation of irrigation potential of ACB, and establish methodology for water demand assessment, estimations have been made at two representative locations. These are at Bilate Tena and Arbaminch. At these two locations assessments have been made based on FAO procedures. The procedures and guidelines for estimating irrigation water demand for the research area, and the methods and results of necessary hydro-meteorological data are further elaborated in Seleshi (2000). Table 4 below provides summary of major parameters derived from the computations. Furthermore, a number of additional scenarios (based on various agricultural plans) could be designed on the exploitation of the potential in order to optimize the available resources or conversely to identify the worst scenario which demand largest storage provision.

| Station | Type of Crop | ETo (mm) | CWR (mm) | Total Rain (mm) | Effective Rain (mm) | IR (mm/period) | FWS (l/s/ha) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Arbaminch | Cotton | 578.55 | 487.73 | 439.53 | 368.92 | 158.69 | [0.17] |
| Arbaminch | Cotton | 578.55 | 487.73 | 239.2 | 210.98 | 322.06 | [0.35] |
| Arbaminch | Maize | 487.75 | 421.54 | 361.86 | 299.42 | 147.2 | [0.20] |
| Arbaminch | Maize | 487.75 | 421.54 | 270.27 | 232.85 | 202.4 | [0.28] |
| Bilate | Cotton | 556.34 | 464.5 | 449.2 | 383.56 | 123.87 | [0.14] |
| Bilate | Cotton | 556.34 | 464.5 | 224.6 | 207 | 271.65 | [0.30] |
| Bilate | Maize | 483.77 | 416.04 | 361.18 | 307.91 | 131.12 | [0.18] |
| Bilate | Maize | 483.77 | 416.04 | 205.94 | 186.33 | 234.1 | [0.32] |

Table 4: Summary of Irrigation Requirements at Two Representative Potential Sites, Two Representative Crops and Two Rainfall Classes.

Where

CWR	: Crop Water Requirement

FWS	: Field Water Supply assuming continuous supply and irrigation at efficiency of 70%

IR	: Irrigation Requirement

TWSI	: Total Water Supply requirement at Intake/diversion

The planting date in the above schedule is chosen in such a way that the planting date coincides with the normal planting period of the region and to provide minimum crop water requirement. As the purpose here is to determine the volume of water needed to develop the available land potential, based on representative crop at representative site, assuming that other elements such as soil, water quality, etc., are suitable and to investigate the overall impact of development through water balance model, the provided result suffice.

In order to determine the gross water demand, result at the two selected sites should be extrapolated based on total water consumption per unit area development and considering conveyance efficiency and cropping intensity. The cropping type may be assumed equal proportion of the two types of crop covering the developable area. Accordingly, the total water demands under realization of the potential based on three scenarios are considered. These are:

Scenario 1: No further development (only existing irrigation development)

Scenario 2: Exploitation of irrigation potential based on pre-identified irrigation development sites, similar to data of Table 3, together with already developed area which brings developed area to 35,000 ha by year 2025.

Scenario 3: Exploitation of all potential irrigation areas identified in the previous section, which are associated to river valleys, similar to data of Table 2 which brings developed area to 77,000 ha by year 2025 (anticipated growth of 2,800 ha per year).

Table 5 provides mean total water requirement that should be taken out of river, based on the above scenarios, cropping intensity of only 100%, average irrigation water requirement based on the result of  the two representative stations, 70% irrigation and 75% conveyance efficiency, two crop types covering equal proportion of indicated area and mean rainfall basis. It also assumes 85% of net irrigation area of the identified potential.

| Scenario | Total Area (ha) | Crop | Growing Period (days) | CWR (mm) | IR (mm) | TWSI (106 m³) |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario 1 | 8,000 | Cotton | 150 | 476.12 | 141.28 | 9.149562 |
|  |  | Maize | 120 | 419.09 | 139.16 | 9.012267 |
|  | Total |  |  |  |  | 18.16183 |
| Scenario 2 | 35,000 | Cotton | 150 | 476.12 | 141.28 | 40.02933 |
|  |  | Maize | 120 | 419.09 | 139.16 | 39.42867 |
|  | Total |  |  |  |  | 79.458 |
| Scenario 3 | 77,000 | Cotton | 150 | 476.12 | 141.28 | 88.06453 |
|  |  | Maize | 120 | 419.09 | 139.16 | 86.74307 |
|  | Total |  |  |  |  | 174.8076 |

Table 5: Summarized Estimate of Total Water Demand Under Mean Rainfall for Exploitation of ACB Irrigation Potential.

The above summarized result provides a general view of the water demand that should be abstracted from the rivers or lakes to exploit the irrigation potential of the ACB in a year, assuming single cultivation in a year planted during usual crop planting period. Obviously, in actual design based on multitude of scenarios the total water demand could be different from the above result.

# The Water Balance Model

The water balance model uses results of study on physical characteristics of the lakes such as GIS based information system of the drainage area and bathymetric surveyed physical morphometric characteristics of the lakes. The model uses, a 2-parameter monthly based rainfall runoff model, as a means of data generation for ungauged inflow information. Lake water level data, whos quality is filtered and adjusted is used in the model. Modified rainfall elevation relationship, evaporation from open water surface and evapo-transpiration equations are used to determine hydro-meteorological elements of the water balance components under limited data situation. The basis for model equation is discussed in Seleshi (2000). The step by step procedure for volume based method of simulation procedures can be described as:

- Compute initial parameters such as area and volume from the initial depth as boundary conditions

- Assume, mean area in time period i,  Am,i  = A1,i , where A1,i initial area

- Compute change in volume, ΔVi, from:

ΔVi=Vin,i−Vou,i+(EF⋅pi−PF⋅ei)⋅Al,i

Vin,i and Vou,i  are inflow and outflow for time interval i. Am,i is mean lake area in the particular time interval. The parameters, EF and PF are adjustment factors for evaporation and rainfall.

- Compute final volume in the time interval, V2,i, from:

V2,i=V1,i+ΔVi

- Compute Z2i by iteration from volume capacity curve

- Compute A2,i, from area elevation curve.

- Compute Am,i, from :

Am,i=A1,i+A2,i2

- Repeat from step 2 until acceptable convergence attained.

- For the next time interval Z1,i+1, A1,i+1, V1,i+1 are described by Z2,i, A2,i, V2,i.

Figure 2: Data Input Elements and Interaction of Various Elements in LAKEBAL model

# Use Scenarios and Evaluation of Impact on Lake Water Balance

It can be argued that, the Lakes Abaya and Chamo might disappear or substantially shrink due to the following major reasons. These are:

- Effect of natural condition: decrease in rainfall and runoff, and increase evaporation, which is associated to climatic change.

- Siltation and complete fill up of the water carrying capacity of the Lakes.

- Overuse of water resource for consumptive purposes

- Misuse of the water resource and pollution

Factor 3 will be considered to see the impact of irrigation and results of factors 1 & 2 will be provided for comparison purpose.

  - Impact of Water Resources Exploitation

Due to population growth in water supply demand and the need for more agricultural food production need and growth of water use in irrigation, the consumptive purposes would be expected to grow in future. Given also programs of energy development using small scale hydropower development in the basin, the non-consumptive water resources use is expected to grow. Impact of consumptive uses with respect to water quantity in the Lakes and water level under future possible development potential identified in the previous sections can be investigated. Impact of non-consumptive uses such as in hydropower generation would have no effect on the water quantity. The non consumptive uses with storage have even positive contribution to the water resources quantity residing in the Lakes.

The possible potential uses, which have water quantity reducing consequences, are irrigation and drinking water supply. The drinking water supply demand should be satisfied from surface and ground water resources, and if the full irrigation potential is to be exploited it is logical to assume the demand would be mainly satisfied from surface water running through rivers. Impacts of these water consumptive uses could be analyzed with the developed water balance model. The investigation of the impact in terms of water quantity is helpful to make decisions on the level of exploitation (such as without significantly reducing/affecting the lake size and volume) of the resource.

In order to use the model to evaluate the direct impact of possible future development, the already identified potentials which utilize water for consumptive purposes can be evaluated in the model. In such analysis future necessary information is based on available data, in which past historical records are considered to repeat in similar manner in future. That means, data of rainfall, evaporation, runoff, simulation parameters and etc., are assumed to be having similar magnitude and characteristics as historic records and derived parameters based on historic records.

What is changing or made to vary is water uses which can be seen in various scenarios. For evaluation of impact on possible circumstances three scenarios of water use identified in previous sections together with drinking water supply need and their combinations are investigated. The total basin water demand which are assumed under growth of population, growth of drinking water demand, stagnant livestock, limited growth of urban industrial demand as per previous Chapter as well as irrigation development: without growth, with medium growth and extensive exploitation up to year 2025 are investigated.

The combined scenarios further presuppose that in drinking water supply just 55% comes from surface water resources. Accordingly, the data of the above statements have been computed and the ACB demand growth and ultimate monthly demand distribution by year 2025 are provided in Figure 3 and 4. This demand has composed from the two lake's watersheds, and particular demands with respect to the Lakes are used in the simulation.

Figure 3: Abaya Chamo Sub-Basin Combined Demand Scenarios

Figure 4: ACB Monthly Mean Consumptive Water Demand Distribution as of Year 2025 Under Three Scenarios

After utilizing the above demand scenarios, the water balance model has been executed by considering for Abaya Lake, 1970 data corresponds to year 2000 and with sequences in which data of 1996 to correspond year 2026. For Chamo Lake, 1970 corresponds to 1975 and 1996 corresponds to 2021. Under these transformations of data, the simulation program has been executed, and Figures 5 and 6 provide the results for the two Lakes.

Further conditions used in the simulation include that simulation is undertaken without sediment input and past computed overflow magnitude and inflow in to Chamo from Abaya overflow are considered the same (one can also refine this component, but for all practical purpose it can be considered the same). In order to compare the result, the gauged data, the simulated data based on past simulation result has been composed with the new simulation.

Figure 5:Simulation of Abaya Lake Water Level Forward in Time for Impact Assessment

Figure 6: Simulation of Abaya Lake Water Level Forward in Time for Impact Assessment

Based on the result, Table 6 provides quantitative summaries for impact of water use under three scenarios at the end of indicated simulation time periods. This means under scenario_1, the Abaya Lake is expected to be lower than existing by 0.87m. Under scenario_2, in which the water supply remains as scenario_1, and irrigation development reaches projected 29,000ha in Abaya watershed, under single cropping season and other various condition discussed earlier, the lake water level drops by 1.23m, and etc.

| Lake | Simulation Period | Expected Loss of Lake Water Level Depth (m) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Scenario_1 | Scenario_2 | Scenario_3 |
| Abaya | 2000-2025 | 0.87 | 1.23 | 1.78 |
| Chamo | 2000-2021 | 0.56 | 0.73 | 1.41 |

Table 6: Water Level Depth Loss of Abaya-Chamo Lakes Due to Water Use

As stated earlier, water use scenarios and computation of magnitude could vary significantly from the above depending various factors such as variation of per capita demand consideration, actual population growth, on site crop water requirement, designed cropping intensity, etc.

Analysis of other impacts described previously, such as natural condition, input of sediment have been also carried out and shown that input of sediment leads to shortening of the life of the Lakes. As the lakes are quite shallow, average 8.6m for Abaya and 10.2m for Chamo, the life expectancy of Abaya is 208 years and that of Chamo is 375 years, unless measures against catchment erosion takes place.

# Conclusion & Recommendations

The water resources in the ACB is not abundant and well distributed in time and space. The resource is scarce and the Lakes are highly vulnerable to disappearance if proper corrective measures couldn't be taken. There is a pressing need to utilize the land and water resources potential of the basins due to growing population. If the available water resource in the system is properly managed, it could be effectively utilized for regional development and preservation of this sensitive environment. The following are suggested measures how to reverse the existing detrimental situation and yet utilize the water resources more effectively.

- Program of erosion control measures in the entire watershed system such as forestation, terracing, prevention of overgrazing, avoidance of poor farming practices.

- Use of renewable and sustainable energy sources such as development and use of hydropower, geothermal, bio-energy and sun energy instead of wood mainly as a source of energy for cooking and heating, and thereby reduces deforestation of the watershed and erosion and sediment transport.

- Replacement of wood construction material with other sources such as clay-bricks, soil-cement blocks and stones, there by reduce deforestation and its consequences.

- Development of water and sediment control structures by choosing elevated reservoir sites. There by on one hand sediment are forced to settle in the reservoir or before the reservoir by providing check dams, and on the other hand the stored water is exposed to lower evaporation and the evaporation difference could be employed in to consumptive and non consumptive uses as well as defined releases to flow through the rivers and streams to sustain the aquatic life in the rivers and wetlands leading to the Lakes. The controlled sediment obviously reduces the input of sediment in to the Lakes and thereby the life of the Lakes could be prolonged. The reduction of evaporation due to higher altitude is highly significant and if sufficient size of reservoir could be provided the evaporation difference alone could be used for substantial development of the irrigation potential.

- Increase of agricultural productivity through small to large irrigation schemes and thereby enable food self-sufficiency of the region and enhance socioeconomic development. In irrigation development, existing practice of water misuse and poor management should be corrected through proper training of the users.

- Environmental education of the community through various associations, organization schools, etc.

References

Abebe B. (2000). Dry-Spell Analysis for Studying the Sustainability of Rain-Fed Agriculture in Ethiopia: the Case of Arbaminch Area. Proceedings of 8th  Nile 2002 Conference: Comprehensive Water Resources Development of the Nile Basin: Priorities for the New Millennium (in Press). Addis Ababa, Ethiopia

Dessalegn Rahmato, (1999), Water Resources Development in Ethiopia: Issues of Sustainability and Participation. Forum for Social Studies, Addis Ababa University.

English, M., (1997). Partial Irrigation: A Fundamental Change. Managing Water: Coping With Scarcity and Abundance, Proceedings of Thema A, ASCE, New York

ESTC (Ethiopian Science and Technology Commission), (1994). National Science and Technology Policy, [Amharic], Addis Ababa.

Gibb, A. and Partners, (1987). Gelana Irrigation Project Feasibility Study, Volumes 1-10.

Halcrow, Sir William and Partners Ltd. (1992), Reconnaissance Master Plan for the Development of the Natural Resources of the Rift Valley Lakes Basin, Volume 3, Appendix C., ETH/88/001, Food and Agricutural Organization of the United Nations

IFAD (International Fund for Agricultural Development ), (1987). Staff Appraisal Report. Ethiopia, Special Country Program. Report No. 6393-ET.

Tahal, (Consulting Engineers) and Metaferia (Consulting Engineers PLC), (1998). Gidabo Irrigation Project, Phase I: Preliminary Assessment. Final report. Project Document for MIDROC Ethiopia, PLC.

WAPCOS (Water and Power Consulting Services, India) (1995), Water Resources Development Master Plan for Ethiopia, Final Report. Volumes I to X. Prepared for EVDSA. Addis Ababa, June

World Bank (1973).  Ethiopia: Agriculture Sector Review. Volume II, Annex 11.  Report No. PA-143a. Washington January 15